*From jelle <jelle@cistron.nl>

S. Harrison wrote:

> Maybe you wouldnt mind explaining what the hell a compressor

> actually does? :)

a compresser is basically a controllable amplifier. What it

does is "shrink" the volume of the sound when it get louder;

thus compressing the sound.

imagine this sound (fixed width font pls):

|     .

|   .  .

|  .    .

| .       ` ....

0dB +----------------

short peak, low volume decay. tipical "drum" sound (snare etc)

a compressor makes it into this (more or less):

|

|     __

|  .`   `.

| .       ` ....

0dB +----------------

the result is that you can crank up 0the volume and make the softer

sounds better to hear (decreasing the sounds dynamics).

that's basically it. hope you got it a little bit.

Also, i think i am reinventing the weel, so check out some music

FAQs.

see you,

jelle.

--

jelle  herold

design_my_ASS

video + audio

*From "doug L." <dugster@primenet.com>

"S. Harrison" wrote:

> Maybe you wouldnt mind explaining what the hell a compressor actually

> does? :)

>

> trey

>

> ________________________________________________________

> The Nord Modular Mailing List is a free service of WIZOO

> http://www.wizoo.com

See Mr. Marko's post.

What the hell is a compressor?

It basically creates a closer ratio between audio peaks and

valleys. The peaks -the loud parts are adjusted by a compressor,

making them less loud.  So if your original loud part is -3 db and your

quieter parts are -20 db, the compressor can lower the level of

the loud parts.

So the -3 db peaks, might become, say, -10 db, with compression.

So now you would have peaks at -10 and quiet parts still at -20.

Without compression, you could have raised the peak levels

and only 3 db before clipping/distortion occurs. The result would be

peaks at around 0 db, and those quieter parts would raise to -17 db,

(up from -20)

After compressing, you could raise the overall level, (with peaks

near 0 db)  by 10 db.     the -10 would become close to 0,

AND THE QUIETER PARTS WOULD ALSO BECOME 10 db louder!

Without compression:

Peaks:  -3 db

Valleys: -20 db

can be raised to:

Peaks:  0 db

Valleys: -17 db

WITH compression:

Peaks:  -10 db

Valleys: -20 db

can now be raised to:

Peaks:  0 db

Valleys:  -10 db !!

Raising the levels of the quieter parts is often

a goal in recording. The very beginning of an

instrument's sound is often much louder than

it's following section. Like when an electric guitar

string is picked. (ie a clean Telecaster twang)

A compressor would lower the volume of the loud

picked part, and then you would be able to raise

the entire volume of the guitar, for a more uniform

volume level.   -doug