*From "Ulrich Reuter" <herrreuter@01019freenet.de>

dear masters

I once read that patching two 12db Filters in a row results in a 24 db

filter; adding even more modules (like possible in the NM) should result in

a VERY exactly cut-off frequency.

First question: Is this true in theory?

Second question: Is it true with a NM as well? My impression is that the

curve of damping frequencies starts far before the indicated point, so that

the very edge of the cut-off point is lowered with each added filter.

Oh my english! I can't say it better. A beautiful diagram instead:

Fig.A: "How ulrich wants it":

one filter:          two filters in a row:

I                    I

.........            .........

.                    .

.                   .

.                  .

.                 .

Fig.B: "As Is":

one filter:          two filters in a row:

I                    I

........            ........

..                  ..

.                   .

.                  .

.                 .

-> no chance to build a brick wall?

ulrich

Hi Ulrich,

The theory behind filters is actually so complex that it's better to judge

a filter by it's "sound". But let's shed some more light to the subject. A

standard analog-type lowpass filter as we find them in the NM is based on

selectively shifting the phase of the signal in a frequencydependant way.

If the phaseshift is 180 degrees it cancels out the original signal and

that frequency isn't heard anymore. In the case of a lowpass filter the

shift increases from 0 degrees to 180 degrees if the frequency rises. There

exists no "ideal" filter that suddenly at a given frequency shifts from 0

to 180 degrees, but a good filter tries to make that range as small as

possible. What we call the cornerfrequency or cutoff frequency is the point

where the signal is attenuated -3 dB. Below that frequency the passband

should have no or a very small "ripple", above that frequency it should

drop as fast as possible. There are some types of filtercharacteristics

like Bessel, Butterworth and Chebishev. These are preprogrammed in the

filteralgoritms and control things like the flatness of the passband, the

corner of the cutoff frequency and the curve of the cutoff-slope. I suppose

that these are responsible for the slight difference in sound between

synthesizer filters of different makes. The bad news is that a sharper

cutoff slope gives a bigger ripple in the passband, more irregular

phaseshifts, etc.

The brickwall filter you want would have no ripple, a sharp corner and go

down straight into oblivion at once. Such a filter can be made digitally in

the form of an analysis/resynthesis filter or a transversal filter. The

first type analyzes the frequencycontent of the signal by means of a so

called Fourier-transform. This gives two "graphs" representing the loudness

and phase of each frequencycomponent in the signal. By deleting some of

these components in the graphs and using this information to do a revers

Fourier-transform a brickwallfilter can be made. But for negligable loss in

quality you need a row of NM's to do the required calculations in realtime!

I heard a Capybara system with 28! DSP's can do that job quite nicely.

For transversal filters it's the same story. But besides lot's and lot's of

DSP-power they also need a lot of memory to temporarily store the processed

material and the so called "impulse-response".

Both types also have a definite timedelay of maybe several msec, but if

properly implemented no phases are shifted.

So we might only see this kind of filter in the NM3 or NM4....... If I

remember well Cool Edit Pro has such filters to experiment with. But not

realtime of course.

There are some tricks, of course. If you connect the lowpass and the

highpass output of the 12 dB filter to a crossfade module, we get a new

type of filter with controllable curve. In the midposition the filter is

actually a notch or bandreject filter. This notch becomes less noticable if

the control is moved toward the lowpass or the highpass input, but it is

still there. When cascading a few 12 dB filters with this

outputmodification we can adjust these notchpositions in a way that they

help to make the slope decrease faster just after the cutoff frequency.

Raising the resonance just a bit helps in sharpening the corner. For best

results a calculation should be made, but that's a terrible complex one as

all filters in the cascade will have different settings, compensating for

each others deficiencies, so better forget about that.

The real question is: why a brickwall filter. It hardly pays off to do all

this complex stuff for a sweeping filter. The audible effect would be

negligable.

But for filtering audiomaterial from the inputs it might be nice to

experiment with it. So what you can do is cascade a D-type filter and a 12

dB filter and connect the lowpass and highpass outputs of the 12 dB filter

by means of a crossfade module. Raise the resonance of the 24 dB filter to

a setting of say 24, this increases the corner but also gives a peak. Now

adjust the frequency and LP and HP amount of the 12 dB filter that the

notch both corrects the peak of the 24 dB filter a bit and makes the slope

fall down a bit faster just after the cutoff frequency.

So a common sense approach would be to mimic the effect of a brickwall

filter. After a 24 dB filter we might find out that a strong frequency

component in the suppressed band is still audible. By using the described

notch filter to suppress that component, LP:HP something like 2:1 to start

with, frequency adjustment by ear, the filter will appear to be much

sharper.

So:

1) the soft slope of the 12 dB compensates for the increased resonance of

the 24 dB, giving a sharper corner but reducing the resonance

2) the notch of the 12 dB suppresses the strongest unwanted strong

frequencycomponent in the rejectband of the 24 dB

3) the frequencysetting and the X-fade setting of the 12 dB influence each

other, the frequency shouldn't be set to high or it won't compensate the

higher resonance, the X-fade controls the position of the notch (over a

limited range)

In the attached example you can experiment with the settings. A strong high

harmonic is added to a sawtooth. Playing with the X-fade control and the

frequency control on the 12 dB filter will hopefully makes things clearer.

A diagram could be:

one 24 dB filter                        24 dB with 12 dB with X-fade

-------                                        ---------

\                                                \

\                                               |

\                                             |  /\

\                                           \/   \

\                                               |

\                                             \

Another approach is using a combfilter, useful to remove one frequency and

all of it's harmonics from audiomaterial. It's essentially a delayline with

a negative feedforward. Very popular with spies to remove backgroundsounds

from longrange microphone recordings. In the examplepatch it is used to

completely remove one of the notes from the chord. The phaseresponse of the

combfilter however is so poor that the remaining material sounds distictly

different. Not really suited to remove the vocals from a song, but when

swept it gives a nice phasing effect.

Greetings,

Rob