If you go to the following location you will find a link to a "package" that

includes a couple of patches and a user guide for an emulation of the simple

bass-lead synthesizer found in the Korg Electribe EA-1.

www.delora.com/music/gear.htm

There is no sequencer section (but that is easy enough to add).  This is not

a complex synth nor even a good use of the Nord.  But its a fun little synth

to play and is easy to patch either from a Nord Modular front panel *or from

a midi control box*.  I did it mostly to provide an economical full-featured

monophonic synth that uses less then 50% DSP.  It also is an experiment in

doing a "complete package" for a patch that is really a synthesizer in its

own right.  A complete package would included a comprehensive documented

midi implementation and a user manual.

This may or may not be of use to members of the list but I would appreciate

feedback on the content and how to make releases such as this most usable

even for a user that never patches their modular but only uses downloaded

patches.

Thanks.

Nord Modular E-Tribe A User Guide

Description

This patch is a concise simulation of the bass synthesizer that is currently marketed by Korg known as the Electribe EA-1. This patch provides one-half of what Korg packages together in the product.

The E-Tribe A is a simple two oscillator synthesizer designed to produce a wide range of harmonically interesting lead and bass sounds. The basic patch is configured to be used monophonically. However, given its relatively low usage of DSP resources it can also be used effectively as a polyphonic instrument.

Feature-wise the synthesizer consists of two oscillators each with a variety of waveforms. These then drive a low pass filter with adjustable sharpness of 12, 18, or 24db. The signal path then feeds the standard envelope controlled amplifier. A final stage of effects processing provides adjustable distortion and two effect sends to drive an external delay and reverb unit as a way of example.

Portamento is provided and is triggered through legato play. Adjustments are available for determining the effect that velocity has on both amplitude and filter dynamics.

Two envelope generators are provided, which is more then the standard Korg unit. The first is a simple attack-decay/release (two time constants) which is used to drive the filter cutoff frequency as well as providing optional pulse width modulation of oscillator 1. A second is a full ADSR envelope for volume control but, aside from its release setting, is basically hidden unless the user decides to program sounds using the Nord Modular's edit features.

Various interactions between the two oscillators create rich harmonic starting material beyond the normal waveforms. Both oscillators offer switchable waveforms SAW, SINE, TRIANGLE, and PULSE. OSC2 PULSE is fixed at 50% duty cycle while OSC1 has an adjustment for pulse width as well as pulse width modulation. The output of each oscillator drives a balance control that lets the user selected the amount of each oscillator that feeds the filter.

The two oscillators are both driven by the same frequency control so they will track notes played. OSC2 features both an offset control and a fine control that lets the two

oscillators be detuned. The offset control lets the oscillators be set to fixed intervals and also plays an important role when combined with the cross modulation

capabilities.

Three cross modulations can be selected. The first is SYNC where OSC2 is hard sync'ed to OSC1. By altering the frequency interval between the two classic sync

waveforms can be achieved.

The second is ring modulation. OSC1 modulates OSC2 amplitude and creates resulting AM spectrum. The modulation is not balanced by design so the center frequency component of OSC2 will be present. This can be adjusted if desired within the patch through the editor but has the side effect of making OSC2 inaudible when it is not in the AM mode.

The final is linear FM of OSC2 by OSC1. A modulation depth control allows setting of the modulation index.

Plus Version

I have a standard way of providing remote control and editing of all my Nord patches and have applied the same to the Electribe-A in the "Plus" version patch. These

features consume a little bit of DSP and some module count but add some useful features.

The biggest addition is that the critical timbre parameters all now have relative adjustments. This feature is heavily used in Yamaha gear is useful for modifying a patch in a simple and predictable manner. The standard that I follow (see MIDI Chart) defines eight parameters all of which have no effect when set to the value of 64. Above 64 there is an increase in that parameter over the default patched value, below 64 it is decreased. Parameters included with relative adjustments are: Attack, Decay, Release (for the hidden AMP Envelope); Decay for the Filter envelope; filter envelope amount; filter cutoff and resonance; and an overall "timbre variation" adjustment that is wired through a morph controller to allow you to set the degree and nature of varying the timbre.

The Plus version still manages to come in under 50% DSP which was the goal for this emulation so that two voices could be done with one Micromodular or one slot of the

full Modular.

Description of Parameters (front panel controls shown in parentheses)

These are arranged in module order from top to bottom, left to right. Parameters in Italics are not generally intended for user adjustment.

DYNAMICS

Vel. Sen. - determines how Expression control affects volume. In the off position (full counter clockwise) Expression has no effect and volume is not attenuated. Full clockwise and Expression has full effect. Use this setting to allow some degree of control over dynamics with Expression but not enough so that Express can turn it full off. All other module parameters should not be adjusted

FILTER ACCENT

Vel. Sen. - determines how Velocity control affects filter envelope amount. In the off position (full counter clockwise) Velocity has no effect and the EG affects the filter normally. Full clockwise and Velocity has full effect, causing the amplitude of the filter EG depth to track Velocity exactly. This setting can be used to determine how much accented notes affect filter cutoff.

L. Gain - Sets Velocity curve below keyboard breakpoint

Brk. Pt. - Which note defines upper and lower.

R. Gain - Sets Velocity curve above keyboard breakpoint

AMP ACCENT

Vel. Sen. - determines how Velocity control affects AMP envelope amount. In the off position (full counter clockwise) Velocity has no effect and the AMP envelope operates normally. Full clockwise and Velocity has full effect, causing the amplitude of the AMP envelope depth to track Velocity exactly. This setting can be used to determine how much accented notes affect volume.

L. Gain - Sets Velocity curve below keyboard breakpoint

Brk. Pt. - Which note defines upper and lower.

R. Gain - Sets Velocity curve above keyboard breakpoint

EXPRESSION

Level - MIDI controlled Expression input

MORPH

Morph1 (Timbre Adjust). - morphing control using Defined Timbre relative adjustment

Morph2 (ModWheel). - morphing control using ModWheel. Intended for controlling "motion" aspects of a patch

Morph3 (FootController). - morphing control using Foot Controller. Intended for controlling timbral character of patch

Morph4 (AfterTouch). - morphing control using After Touch. Used to control effects aspects of patch

TUNE

Level - Pitch offset in semitones

PORTAMENTO (GLIDE)

Level - Portamento rate

ACCENT

Fade - This is a bipolar control. In the middle setting (MIDI Value = 64) the velocity received affects the patch, as determined by Filter. Accent and AMP Accent and Filter Cutoff Accent, with maximum dynamic range. Increasing the value diminishes the dynamic range while gradually moving the mid-value up towards full velocity. Lowering the value also diminishes the dynamic range until at a value of zero there is no velocity effect and the effective velocity sent for further processing is mid-value (64).

OSC1

Coarse Frequency - determines the tuning of both OSC1 and OSC2

Fine Frequency - sets fine frequency for both OSC1 and OSC2

KBT - should be off as pitch from keyboard is routed through Glide.

Wave Select (OSC1 WAVE) - OFF, SINE, TRIANGLE, SAW, PULSE

Pitch MA1 - should be full on as this is the pitch control from the glide circuit. Adjust to less then full if normal key-tracking is not desired.

Pitch MA2 - should be full on as this is the input from the instrument TUNE control.

PWM - should be full on as it is driven by external pulse width controls.

Pulse Width (OSC1 PW)

Level - determines starting pulse width of OSC1 PULSE output

PWM

Lin - Should not be adjusted

Inv1 - Should not be adjusted

Mix1 - Amount of EG applied to modulate pulse width of OSC1

Inv2 - Should not be adjusted

Mix2 - Should not be adjusted

OSC2

Detune (OSC2 PITCH OFFSET) - Coarse adjust of ration between OSC1 and OSC2 frequency

Fine (DETUNE) - fine adjustment of frequency ratio between OSC1 and OSC2

Wave Select (OSC2 WAVE) - OFF, SINE, TRIANGLE, SAW, PULSE

FMA (FM DEPTH) - FM depth control

OSC BALANCE

Balance MA - How much balance modulation from EG

Balance (OSC BAL) - OSC1 to OSC2 mix. Full counterclockwise is OSC1 only, Full clockwise is OSC2 only

OSC MOD

Switch1 (SYN) - Enable OSC2 to be synced by OSC1

Switch2 (AM) - Enable OSC2 to be amplitude modulated by OSC1

Switch3 (FM) - Enable OSC2 to be frequency modulated by OSC1

Switch4 (OFF) - No modulation

FILTER

Frequency (CUTOFF) - filter cutoff frequency

Res (RES) - Filter resonance

KBT - filter keyboard tracking. Normally off.

Type Select - OFF, LP, BP, HP, BR

db/oct - 12db/OCT, 24db/OCT

Freq MA1 - envelope depth scaling controlled via MIDI.

Freq MA2 - should be full on as this is the input from velocity depth

EG DEPTH

Lin - Should not be adjusted

Inv1 - Should not be adjusted

Mix1 - Amount of EG applied to filter. 12 O'clock position is off. Full counterclockwise is full negative modulation, full clockwise is full positive modulation

Inv2 - Should not be adjusted

Mix2 - Should not be adjusted

EG

Attack - EG Attack Rate

Hold - EG Hold Time (normally 0.5msec)

Decay - EG Decay Rate

Decay Mod - EG Decay Rate relative adjust input

AMP EG

Attack - AMP EG Attack Rate

Decay - AMP EG Decay Rate

Sustain - AMP EG Sustain Level

Release - AMP EG Release Rate

Attack Mod - AMP EG Attack Rate relative adjust input

Decay Mod - AMP EG Decay Rate relative adjust input

Sustain Mod - AMP EG Sustain Level force on via MIDI (Gate Mode)

Release Mod - AMP EG Release Rate relative adjust input

DISTORTION

Overdrive MA - Amount of modulation provided by EG for distortion

Overdrive - Amount of distortion

VOLUME

Lin - Should not be adjusted

Inv1 - Should not be adjusted

Mix1 (Volume) - Volume adjustment.

Inv2 - Should not be adjusted

Mix2 - Should not be adjusted

REVERB

Level - FX Send level to OUT4

DELAY

Level - FX Send level to OUT3

PAN

Pan - Left-Right Adjust

OUT

Level - Dry output level adjust - not normally adjusted