Hi All,

To complete things, here are tuning systems for 5, 7 and 12 notes /octave. I set them equally tempered, but that can be changed, of course.

I've also been working on a scale with 19 notes / octave - but can’t get it right. Give it up for now, added the patch, but please note that it's a failure really.

Someone, I forgot who, remarked that the scales presented so far are all octave based - this is true, but it could be easily changed by adding an amplifier module after the main note output (on the S&H module) so the octave can be re-scaled at will. This however will also require you to

re-tune all other tuning knobs.

A good thing to know is that the scaler modules (the ones that follow the discriminator chain) have a sensitivity of an octave / 96 steps, or a semi tone / 8 steps, or 12.5 cents / step.

I typed some info into the floating notes for easy reference (see tools/notes floater). In the balaphone patch made by Kofi I noticed him using the gate output of the keyboard module to trigger the envelope generators. Normally this will not be a problem, but in general it would be a better idea to use the delayed gate from the module marked trig&hold (as the note output will only be valid after the rising edge of that module’s output occurred).

Jan.