STEPMAN V35 DEMO  COPYRIGHT BY J.BAARS AND BFX 1988 ` 2008

V35 IS A VERY PRELIMANARY DEMO, NOT ALL THE FUNCTIONS ARE WORKING OR INTEGRATED YET.

THIS IS A FIRST PEEK INTO THE STEPMAN SEQUENCER.

------------------------------------------------------------------------------------------------

ALLREADY TESTED AND WORKING, BUT NOT INTERGRATED YET..............

Channel 1 , 2 , 3 (the CV output channels) , channel 7 ( the switch / gates output channel)

external clock in/output

volumes and pan for the midi channels ( 4 , 5 , 6)

preset memory , save /load presets ( this function will only be available on the commercial version.)

--------------------------------------------------------------------------------------------------

still in development,

link sequences, a continue button and some othe stuff.

--------------------------------------------------------------------------------------------------

The stepman is a 16 step 7 track step sequencer.

the tracks 1, 2 and 3 are CV /gate output ( with a interface on the lpt port) tracks

track 7 is a multy purpose switch track , the 8 binary outputs are available on the lpt interface.

the tracks 4, 5, and 6 are midi tracks, they send the midi to the internal wavesynt of your soundcard and to the midi output

at address 300 ( the nomal common used address for midi out)

THE hard and software;........

the stepman is tested in a real dos environment, windows 3.1, 95, 98 (dosbox, select fullscreen, 50 lines in the options menu)***

not yet tested in XP , Vista.

preferred is Freedos.

***the dos options menu in windows,...

***rightclick on the stepman v35.exe   , select propaties, screen , select fuul screen, select 50 lines.

a shortcut will be created by windows after clicking on the OK button, this shortcut can be used to start the programm.

ater startup fist the progam asks to enter the resets for chan 1 to 6  ( reset number followed by enter)

select tracks

key z > chan.1,  x> chan.2 , c>chan 3, v> chan4, a>chan 5, s>chan 6, g> chan 7.

select resets , all channels may have different resets.

tr1  key q, tr2  key w, 3 e, 4 r, 5 t, 6 y, 7 b.

note # select,...

note #0 tunrs the gate off, 1 to 127 is gate on, output CV controll voltages , midi note numbers.

keys,...

N note up 1 step, M note down 1 step, H note up 10 steps, J note down 10 steps.

midi,...

select midi track,

select with key U the midi channel ( 10 for percussion an d drums)

select sound with key P (up) , O (down)

if ready selecting, push enter to transfer to channel.

START STOP SEQUENCER, spacebar

Speed internal clock F up, D down.

Int / ext clock select , ecc . key. do not use without a ext. clock connected, the program will stop.

////////////////////////////////////////////////////

_